Texas Instruments

IBIS Model Correlation Report

ADS8482 IBIS ver. - 1.0 Corr_Report ver. – 1.0

[Shiju Abraham]

[01/21/2011]

This correlation report contains Datasheet correlation, Measurement correlation and IBIS vs. Spice correlation of the part ADS8482.

IBIS CORRELATION REPORT

Design ID:  ADS8482

IBIS File Name and Version: ads8482.ibs v1.0

Correlation Report Name and Version: ADS8482_Wipro_CorrRpt.doc

IBIS Checklist name and version: ADS8482_Wipro_CheckList.xls

Available package types: VQFN

Marketing part Number(s): ADS8482IBRGZR, ADS8482IBRGZRG4, ADS8482IBRGZT, ADS8482IBRGZTG4, ADS8482IRGZR, ADS8482IRGZRG4, ADS8482IRGZT, ADS8482IRGZTG4

IBIS Zip File Name: ads8482.zip, zip contains IBIS checklist, model & Correlation report

Date: January 21, 2011

Datasheet Title: 18 Bit 1MSPS Parallel ADC W/Ref, Pseudo Bipolar, Fully Differential Input

Datasheet Revision: SLAS386A–JULY 2005–REVISED JUNE 2006

Datasheet Link: http://focus.ti.com/lit/ds/symlink/ads8482.pdf

Contact IBIS modeling Support at elab_scm_ibis@list.ti.com for questions

Note: ADS8482 is relative part of ADS8481

IBIS MODEL CORRELATION

A. Datasheet Correlation

- Compare c_comp with datasheet’s input capacitance spec. Table provides data comparing c_comp for all models and all package combinations

Component Name: ADS8482RGZ

| Model Name |  | IBIS |  |  | Datasheet |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Min(pF) | Typ(pF) | Max(pF) | Min | Typ | Max |
| input_3p3 | C_comp | 0.8568 | 0.8826 | 0.9164 | NA | NA | NA |
|  | C_package | 0.13533 | 0.14658 | 0.15866 | NA | NA | NA |
|  | C_total | 0.99213 | 1.02918 | 1.07506 | NA | NA | NA |
| input_5 | C_comp | 0.8085 | 0.8321 | 0.8631 | NA | NA | NA |
|  | C_package | 0.13533 | 0.14658 | 0.15866 | NA | NA | NA |
|  | C_total | 0.94383 | 0.97868 | 1.02176 | NA | NA | NA |
| convst_3p3 | C_comp | 1.124 | 1.158 | 1.193 | NA | NA | NA |
|  | C_package | 0.18224 | 0.18224 | 0.18224 | NA | NA | NA |
|  | C_total | 1.30624 | 1.34024 | 1.37524 | NA | NA | NA |
| convst_5 | C_comp | 1.073 | 1.105 | 1.138 | NA | NA | NA |
|  | C_package | 0.18224 | 0.18224 | 0.18224 | NA | NA | NA |
|  | C_total | 1.25524 | 1.28724 | 1.32024 | NA | NA | NA |
| busy_3p3 | C_comp | 1.633 | 2.118 | 2.638 | NA | NA | NA |
|  | C_package | 0.15594 | 0.15594 | 0.15594 | NA | NA | NA |
|  | C_total | 1.78894 | 2.27394 | 2.79394 | NA | NA | NA |
| busy_5 | C_comp | 1.553 | 1.989 | 2.445 | NA | NA | NA |
|  | C_package | 0.15594 | 0.15594 | 0.15594 | NA | NA | NA |
|  | C_total | 1.70894 | 1.14494 | 2.60094 | NA | NA | NA |
| db_3p3 | C_comp | 1.72 | 2.206 | 2.744 | NA | NA | NA |
|  | C_package | 0.13804 | 0.15419 | 0.17209 | NA | NA | NA |
|  | C_total | 1.85804 | 2.36019 | 2.91609 | NA | NA | NA |
| db_5 | C_comp | 1.642 | 2.082 | 2.539 | NA | NA | NA |
|  | C_package | 0.13804 | 0.15419 | 0.17209 | NA | NA | NA |
|  | C_total | 1.78004 | 2.23619 | 2.71109 | NA | NA | NA |

- If slew rate specifications (Rise slew and Fall slew) are available, table compares slew rates from SPICE simulation to that of Datasheet.

| Model Name | Slew rate | IBIS [Ramp] |  |  | Datasheet |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Min | Typ | Max | Min | Typ | Max |
| busy_3p3 | Rising(V/ns) | 0.591584/2.32234 | 0.887057/1.33633 | 1.18783/8.48849 | NA | NA | NA |
|  | Falling(V/ns) | 0.475367/2.52494 | 0.845912/1.55421 | 1.19342/9.8449 | NA | NA | NA |
| busy_5 | Rising(V/ns) | 1.27416/1.57095 | 1.56531/1.00139 | 1.86537/7.09787 | NA | NA | NA |
|  | Falling(V/ns) | 1.00576/1.70638 | 1.47069/1.13016 | 1.82562/7.653 | NA | NA | NA |
| db_3p3 | Rising(V/ns) | 0.587642/2.29238 | 0.886173/1.32675 | 1.18781/0.83853 | NA | NA | NA |
|  | Falling(V/ns) | 0.461379/3.05001 | 0.844308/1.9655 | 1.1933/1.24375 | NA | NA | NA |
| db_5 | Rising(V/ns) | 1.27123/1.56137 | 1.56464/0.993778 | 1.86541/0.70146 | NA | NA | NA |
|  | Falling(V/ns) | 0.995787/2.14411 | 1.46902/1.43454 | 1.82539/0.970001 | NA | NA | NA |

B. Measurement Correlation

- For Output or I/O model compare IOH and IOL data from measurements to those of IBIS model. Measurement data and simulations in SPICE are compared across Process, Voltage and Temperature corners. Any special conditions used for simulating or measurements need to be included.

Simulation Tools and version used:

Measurement conditions used:

Model simulation conditions used:

  - Measured IOH vs. IBIS IOH

  - Measured IOL vs. IBIS IOL

- Compare IBIS C_comp  with measured C_comp. Table provides comparison for all models and all package types

Component Name:

| Model Name |  | IBIS |  |  | Measurement |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Min | Typ | Max | Min | Typ | Max |
|  | C_comp |  |  |  |  |  |  |
| Input | C_package |  |  |  |  |  |  |
|  | C_total |  |  |  |  |  |  |
|  | C_comp |  |  |  |  |  |  |
| Output | C_package |  |  |  |  |  |  |
|  | C_total |  |  |  |  |  |  |

- Clamp data if available, include for all models

  - GND CLAMP data compared between IBIS and Silicon measurements. Sweep between –VCC to 2*VCC  (if possible).

- If slew rate measurements (Rise slew and Fall slew) are available, table compares slew rates from SPICE simulation to that of measurements.

| Model Name | Slew rate | IBIS [Ramp] |  |  | Measurements |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Min | Typ | Max | Min | Typ | Max |
| Output | Rising(V/ns) |  |  |  |  |  |  |
|  | Falling(V/ns) |  |  |  |  |  |  |

Note: This data is taken from the Parent Part ADS8481.

C. IBIS vs. SPICE Correlation

Case 1: db_3p3

SPICE engine used to generate and correlate IBIS models:

P.S: {SPICE type could be Spectre, TISPICE, TISPICED, HSPICE, HSPICED}

Generation:

TISPICE

Correlation:

SPICE Netlist Simulation – TISPICE

IBIS Model Simulation – HSPICE

- For all buffers correlate V-T transient simulations using IBIS(B-element) in HSPICED and SPICE netlist to ensure correlation. Include screen shot of test setup used for correlation. SimDE test setup with recommended load conditions may be used.

  - Use node naming convention of IBIS_<corner> and SPICE_<corner>.

  - Run simulation for all corner cases and maximum allowable speed grade, if multiple speed grades or drive strengths are allowed, please include separate correlation waveforms for each case. Include waveforms here. Re-use C.1.i and C.1.ii as needed for multiple buffers and or configurations. Include any correlation metrics if available.

DAI values

IBIS vs. SPICE

Rising edge correlation

Falling edge correlation

  - Test Frequency : 1 MHz

Case 2: db_5

SPICE engine used to generate and correlate IBIS models:

P.S: {SPICE type could be Spectre, TISPICE, TISPICED, HSPICE, HSPICED}

Generation:

TISPICE

Correlation:

SPICE Netlist Simulation – TISPICE

IBIS Model Simulation – HSPICE

- For all buffers correlate V-T transient simulations using IBIS(B-element) in HSPICED and SPICE netlist to ensure correlation. Include screen shot of test setup used for correlation. SimDE test setup with recommended load conditions may be used.

- Use node naming convention of IBIS_<corner> and SPICE_<corner>.

- Run simulation for all corner cases and maximum allowable speed grade, if multiple speed grades or drive strengths are allowed, please include separate correlation waveforms for each case. Include waveforms here. Re-use C.1.i and C.1.ii as needed for multiple buffers and or configurations. Include any correlation metrics if available.

DAI values

IBIS vs. SPICE

Rising edge correlation

Falling Edge Correlation

- Test Frequency : 1 MHz

Case 3: busy_3p3

SPICE engine used to generate and correlate IBIS models:

P.S: {SPICE type could be Spectre, TISPICE, TISPICED, HSPICE, HSPICED}

Generation:

TISPICE

Correlation:

SPICE Netlist Simulation – TISPICE

IBIS Model Simulation – HSPICE

- For all buffers correlate V-T transient simulations using IBIS(B-element) in HSPICED and SPICE netlist to ensure correlation. Include screen shot of test setup used for correlation. SimDE test setup with recommended load conditions may be used.

- Use node naming convention of IBIS_<corner> and SPICE_<corner>.

- Run simulation for all corner cases and maximum allowable speed grade, if multiple speed grades or drive strengths are allowed, please include separate correlation waveforms for each case. Include waveforms here. Re-use C.1.i and C.1.ii as needed for multiple buffers and or configurations. Include any correlation metrics if available.

DAI values

IBIS vs. SPICE

Rising edge correlation

Falling edge correlation

- Test Frequency : 1MHz

Case 4: busy_5

SPICE engine used to generate and correlate IBIS models:

P.S: {SPICE type could be Spectre, TISPICE, TISPICED, HSPICE, HSPICED}

Generation:

TISPICE

Correlation:

SPICE Netlist Simulation – TISPICE

IBIS Model Simulation – HSPICE

- For all buffers correlate V-T transient simulations using IBIS(B-element) in HSPICED and SPICE netlist to ensure correlation. Include screen shot of test setup used for correlation. SimDE test setup with recommended load conditions may be used.

- Use node naming convention of IBIS_<corner> and SPICE_<corner>.

- Run simulation for all corner cases and maximum allowable speed grade, if multiple speed grades or drive strengths are allowed, please include separate correlation waveforms for each case. Include waveforms here. Re-use C.1.i and C.1.ii as needed for multiple buffers and or configurations. Include any correlation metrics if available.

DAI values

IBIS vs. SPICE

Rising edge correlation

Falling edge correlation

- Test Frequency : 1MHz

Case 5: input_3p3

DAI values

- Test Frequency : 1MHz

Case 6: input_5

DAI values

- Test Frequency : 1MHz

Case 7: convst_3p3

DAI values

- Test Frequency : 1MHz

Case 8:convst_5

DAI values

- Test Frequency: 1MHz

D. Correlation Report Revision History:

Rev 1.0: 01/21/2011

-First release from Wipro. This part is relative part of ADS8481.

Do not edit below this

E. TIPS:

-If data is unavailable for any of the sections, please type “UA”. If a section is not applicable for a model category please type “NA”.

E. Template Revision History:

Rev 1.0: 01/22/2009

- Initial revision based on IBIS Quality Spec 1.0

Rev 1.1: 07/10/2009

-Template updated based on IBIS Quality Spec 1.1as (draft) dated 06/02/2009

-New Quality Checklist added

Rev 1.2: 12/30/2009

-Template updated to meet IBIS Quality Spec 2.0 ratified on 10/30/2009

Rev 2.0: 02/15/2010

-Renamed template as IBIS_Correlation_Report_template_2p0.doc

-Removed IBIS Model Checklist from the report. This report is only for correlation               purposes. Use IBIS Model Checklist for Quality Analysis

Rev 2.1: 06/02/2010

-Template updated to meet AEDC requirements

Rev 2.2: 07/08/2010

-Updated file naming convention for correlation report and quality checklist

Rev 2.3: 09/02/2010

-Updated how correlation waveforms should convey clear edge correlation using an example waveform correlation picture. This example correlation picture should be deleted in the final correlation report.

-Updated file naming convention for correlation report and quality checklist.

Rev 2.4: 09/15/2010

-Updated correlation waveforms format both for single-ended and differential buffers