User Note:

Please be aware that all the standard CR16MCS9 parts (except for the part on the evaluation board) are all programmed with National’s ISP monitor by default. However, in order to communicate with the ISYSTEM WinIDEA debugger, you have to run the ISYSTEM debugger on the CR16MCS9 part. If you want to install the ISYSTEM debugger on a standard CR16MCS9 part, please follow the instruction on the next page.

How to update a standard CR16MCS9 with the iSYSTEM monitor?

-----------------------------------------------------------------------

First of all follow the procedure to install the National ISP Monitor:

Windows 98/2000/ME - Install ISP v3.

Run \Installs\Isp monitor\Isp3\setup.exe.

Windows 95         - Install ISP v3.

Run \Installs\Isp monitor\Isp3\Setup.exe.

If this does not work, install ISP v2.

Run \Installs\Isp monitor\Isp2\ISP2Setup.exe.

Note, however, that not all the functions in ISP v2 are supported

by the CR16 evaluation board. The ISP monitor can be used only for

in-system programming.

---------------------------------------------------------------------------------------------------------------------------------

Then follow the steps to update with the ISYSTEM debugger:

Please go to the site  http://www.national.com/appinfo/cr16/ISP.html or email your local distributor to get the two files required during update: “upd_cb.xpr” and  “monitor.xpr”

1. Install new part into eval board

2. Set ENV jumper to ISP mode

3. Power up the board (device enters ISP mode and executes the standard factory programmed

ISP code)

4. Program the file "upd_cb.xpr" into the on-chip flash program memory (address 0) of the

device and subsequenctly clear the empty bit by means of ISP Monitor.

- select chiptype "CANnonball"

- Connect

- open file

- program

- clear EMPTY bit in Memory Protection menu

- disconnect (same as connect)

5. Set the ENV jumper to IRE mode.

6. Press RESET button on eval board

(device enters IRE mode and executes the program upd_cb from address 0).

7. Program the file "monitor.xpr" into the on-chip flash program memory (address E000) of the

device by means of ISP Monitor.

- select chiptype "CANnonball"

- Connect

- open file

- program

8. Set the ENV jumper to ISP mode.

9. Press RESET button on eval board

(device enters ISP mode and executes the iSYSTEM monitor).

10. Done!