DM8127 CSK

Diagnostics User Manual

| CHANGE HISTORY |  |  |  |  |
| --- | --- | --- | --- | --- |
| Version No. | Date | Change Description | Author | Remarks |
| 0.1 | 17-Oct-16 | Created | Antriksh |  |
| 1.0 | 17-Nov-16 | Changed diagnostic Test Procedure section to Diagnostic Flow and updated Test section | Antriksh |  |

| TEMPLATE CHANGE HISTORY |  |  |  |
| --- | --- | --- | --- |
| Version No. | Date | Change Description | Remarks |
| 1.00 | 20-Jul-11 | Initial Baseline |  |

Table of Contents

1 INTRODUCTION	4

1.1 Purpose and Scope	4

1.2 Acronyms	4

2 Overview	5

2.1 Diagnostics Flow	6

2.2 DDR3L Test	16

2.2.1 Test Accessories	16

2.2.2 Test Setup	16

2.2.3 Test Execution	16

2.3 Nand Flash Test	17

2.3.1 Test Accessories	17

2.3.2 Test Setup	17

2.3.3 Test Execution	17

2.4 SD Card Test	18

2.4.1 Test Accessories	18

2.4.2 Test Setup	18

2.4.3 Test Execution	18

2.5 Ethernet Test	19

2.5.1 Test Accessories	19

2.5.2 Test Setup	19

2.5.3 Test Execution	19

2.6 Audio Test	20

2.6.1 Test Accessories	20

2.6.2 Test Setup	20

2.6.3 Test Execution	20

2.7 BT Test	21

2.7.1 Test Accessories	21

2.7.2 Test Setup	21

2.7.3 Test Execution	21

2.8 USB Test	21

2.8.1 Test Accessories	21

2.8.2 Test Setup	21

2.8.3 Test Execution	21

2.9 LED Test	22

2.9.1 Test Accessories	22

2.9.2 Test Setup	22

2.9.3 Test Execution	22

2.10 Switch Test	23

2.10.1 Test Accessories	23

2.10.2 Test Setup	23

2.10.3 Test Execution	23

2.11 WLAN Test	24

2.11.1 Test Accessories	24

2.11.2 Test Setup	24

2.11.3 Test Execution	24

2.12 Camera Test	24

2.12.1 Test Accessories	24

2.12.2 Test Setup	24

2.12.3 Test Execution	24

2.13 HDMI Test	25

2.13.1 Test Accessories	25

2.13.2 Test Setup	25

2.13.3 Test Execution	25

2.14 Composite Video Test	25

2.14.1 Test Accessories	25

2.14.2 Test Setup	25

2.14.3 Test Execution	25

3 Appendix	26

3.1 Creating Ethernet Loopback Cable	26

# INTRODUCTION

## Purpose and Scope

This document provides the details of DM8127 diagnostic test software suite. The scope of this document is limited to the diagnostic software, its setup, compilation and test procedures on the DM8127. It is assumed that the audience of this document is familiar with using the DM8127 CSK Board and has access to the DM8127 reference  document(TRM).

## Acronyms

| Acronyms | Description |
| --- | --- |
| DDR | Double Data Rate |
| LED | Light Emitting Diode |
| SD | Secure Digital |
| SoC | System on Chip |
| TRM | Technical Reference Manual |
| UART | Universal Asynchronous Receiver/Transmitter |
| USB | Universal Serial Bus |
| CCS | Code Composer Studio |
| IO | Input/Output |
| GPIO | General purpose input/output |
| WLAN | Wireless local area network |
| BT | Bluetooth |

# Overview

DM8127 CSK diagnostics software provides a set of interface tests to validate the DM8127 CSK hardware. Diagnostic tests enable quick verification of the interface connectivity and basic functionality before starting the development on DM8127. Diagnostic software also serves as reference code for low level hardware configurations of the DM8127 CSK and SoC. Diagnostic test suite is designed to execute from CCS on DM8127 CSK along with Carrier card. Diagnostic tests use UART serial interface of SoC to output the test results and accept the user inputs.

Below picture shows the major interfaces on the DM8127 Board.

FAULT INDICATION LED (LD2)

POWER LED

(LD1)

ON/OFF SWITCH (SW1)

5V DC

JACK (J1)

Boot mode Switch (SW1)

Reverse Voltage Indication LED (LD3)

CSI Camera (Expansion) Connector (J15)

TMDSCSK8127 Module

TVOUT

CONN (J13)

ETHERNET Connector (J8)

Parallel Camera Module Connector (J7)

DEBUG USB CONN (J11)

HDMI Connector (J6)

CLIENT USB CONN (J12)

LINE-OUT

JACK (J9)

LINE-IN

JACK (J10)

## Diagnostics Flow

- Connect 5v adapter to DM8127 CSK

- Connect micro USB cable between the micro USB port of carrier card and host PC

- Connect all the Test accessories required for Interface testing(for e.g ethernet loopback cable,audio headset,HDMI cable e.t.c)

- Power “ON” the board

- Open CCS

- Import diagnostic project (Skip this step and go to step 8 to use pre-built binary)

- Go to project -> Import CCS Projects

- Browse to the path to import diagnostic project dm8127_csk_diagnostics\build\dm8127_csk_diagnostics

- Click finish

Note : Finish options highlights once project is available in a selected path.

Once Project is Imported successfully, Press right click button and click on Build project.

Successful execution of build will create diagnostic test binaries(i.e dm388csk_diagnostic.out) under the Path /dm8127csk_diagnostic/Debug/dm8127csk_diagnostic.out as shown in above figure.

- Import and launch the DM8127 target configuration file

A.  To Import the configuration file Go to view -> Target configurations.

B.  Right click on the target configurations widow and import the DM8127.ccxml file

C.  To launch the target configuration, right click on the configuration file and click on

Launch selected configuration.

- Make sure DM8127.gel file is loaded on to Cortex- A8

A.  To load the GEL file go to Tools -> GEL Files.

B.  To select the GEL file, Go to Gel file window and Right click on the GEL

Files(Cortex_A8) and select the gel  Files.

- Make target connection to Cortex- A8 Processor

- Reset the CPU after connecting the target.

- Click  on Run -> Reset -> CPU Reset

- Open serial console (Ex: Teraterm) on host PC, connect to COM port on which BoosterPack Debug port is connected and setup for following configurations

    - Baud rate -    115200

    - Data length -  8 bit

    - Parity -          None

    - Stop bits -      1

    - Flow control -  None

- Load and run dm8127csk_diag.out

A. To load the program, go to Run -> Load -> Load  Program

B.  Once .out file is get loaded press F8 to run the program which should display the diagnostic test menu in serial console as shown below

======================================================

DM8127 CSK Hardware Diagnostic Tests

======================================================

Options    Diagnostic Tests		  PASS		FAIL

-------      ----------------		             ----		 ----

0 - 	     Auto Run All Tests           	    0		  0

1 - 	     DDR Test                     		    0		  0

2 - 	     NAND Flash Test             	    0		  0

3 - 	     SD Card Test                 	 	    0		  0

4 - 	     Ethernet Test                		    0		  0

5 - 	     Audio Test                   	  	    0		  0

6 - 	     Bluetooth Test               	  	    0		  0

7 - 	     USB Test                     		    0		  0

8 - 	     LED Test                     	  	    0		  0

9 - 	     Switch Test                  		    0		  0

10 -     WLAN Test                    	  	    0		  0

11 -     Camera Test                  	  	    0		  0

q -	 	Quit

=======================================================

Enter Desired Option :

## DDR3L Test

### Test Accessories

NA

### Test Setup

Default

### Test Execution

This test verifies DDR3L interface access on DM8127 CSK. Below operations are executed as part of this test

- Incremental pattern test

- Fixed pattern 0xA5A5A5A5

- Fixed pattern 0xFFFFFFFFF

Sample DDR3 test log is shown below

===================================

DDR3 Test

===================================

DDR3 test performs Multiple pattern test on the 64KB of the DDR memory

from starting location of the DDR memory

This test write the pattern on the DDR memory verify after

reading the data, In case of any error memory location will display

Carrying out Incremental pattern test for DDR[0]...

Carrying out Fixed pattern 0xA5A5A5A5 for DDR[0]...

Carrying out Fixed pattern 0XFFFFFFFF for DDR[0]...

Carrying out Incremental pattern test for DDR[1]...

Carrying out Fixed pattern 0xA5A5A5A5 for DDR[1]...

Carrying out Fixed pattern 0XFFFFFFFF for DDR[1]…

------------------------------x-----------------------------

## Nand Flash Test

### Test Accessories

NA

### Test Setup

Default

### Test Execution

This test verifies NAND flash interface access on DM8127 CSK. Below operations are executed as part of this test

- Reading NAND flash device information

- Erasing NAND flash blocks

- Writing data to NAND flash page

- Reading data from NAND flash page and comparing the data with written data for correctness

- Verifications of NAND flash write protect

Sample NAND Flash test log is shown below

===================================

NAND Flash Test

===================================

NAND Flash initialize successfully  MFGID = 2c

DEVID = ca

Testing 4 blocks

Erasing NAND blocks...

Total  0 block failed to erase

Programming NAND pages

Comparing data

No Error in data

Write protect disable

Enabling the write protect

Write protect disable

Total  0 block failed to erase

Programming NAND pages

Comparing data

No Error in data

Write protect disable

Disabling the write protect

Write protect disable

Total  0 block failed to erase

Programming NAND pages

Comparing data

No Error in data

NAND flash test performed

--------------------------------x----------------------------------

## SD Card Test

### Test Accessories

SD Card

### Test Setup

Insert SD Card to the DM8127 CSK

### Test Execution

This test verifies SD card interface access on DM8127 CSK. Below operations are executed as part of this test

- SD card detection

- Writing data to a block/sector

- Reading data from a block/sector

- Verification of read/write data

Sample log for SD test is shown below

===================================

SD Card Test

===================================

SD_Card Tests:

The test verifies the command, clock and data lines of sd/mmc

by writing and verifies the data written,

Ensure that the Boot Switches are selected as specified

and SD card is inserted into the Base Board of your EVM

Running SD/MMC test

SD Card Detected

SDMMC: Writing a pattern to Block 1....

Write test passed

SDMMC: Reading from Block 1...

Read test passed

Verify data...Passed

SDMMC Test Passed

-----------------------------x-------------------------------

## Ethernet Test

### Test Accessories

Ethernet loopback cable

### Test Setup

Connect the Ethernet loopback cable to the RJ-45 connector

### Test Execution

This test verifies Ethernet interface access on DM8127 CSK. Below operations are executed as part of this test

- Sending and receiving packet via loopback cable

- Verifying phy internal loopback operation

- Verifying phy external loopback operation

Sample log for Ethernet loopback test is shown below

===================================

Ethernet Test

===================================

PHY found at address 25

Testing PHY internal loopback

Enabling the GMII interface on DM8127.

In GMII mode

Packet verification passed

PHY internal loopback test completed

Testing Ethernet external loopback

Waiting for link...

Link Detected

Enabling the MII interface on DM8127.

In MII mode

Packet verification passed

Ethernet external loopback test completed

-------------------------------x--------------------------------

## Audio Test

### Test Accessories

LINE IN cable & headset

### Test Setup

- Connect the LINE IN cable to audio out of  test  PC and  LINE IN port of DM8127 CSK

- Connect the headset to LINE OUT of DM8127 CSK.

### Test Execution

This test verifies that Audio supplied from test PC through LINE IN cable at LINE IN port  is Coming to Headphone which is connected to LINE OUT of DM8127 CSK.

Sample log for Audio test is shown below

===================================

Audio Test

===================================

+=========================================================+

|*********  AUDIO CODEC AIC3104 TEST ON BASE BOARD  **********			|

|  This test covers connectivity tests to AIC3104 on the base 					|

|  for head-phone, Line-in. For this                       						|

|  ensure the following :-                                    						|

|                                                             							|

|      (a) Head phone connected to the Head-phone jack.       					|

|      (b) Stereo Audio cable feeding into the Line-In jack.  					|

|                                                             							|

| Playing the sine tone on the Head phone                     					|

| Look for the tone on the Head phone for the next 5 seconds. 					|

+=========================================================+

Playing the sine tone the Line-out.

Look for tone being played back on Speakers for next 5 seconds

Codec config done

Loopback of Line-In to the Line-Out.

Feed the stereo audio into the Line in and hear back on speaker.

All configuration & Playback/loopback successfully completed.

However, actual results are to be  judged based on the audio

heard from the head-phone and speakers.

-------------------------------x----------------------------------

## BT Test

### Test Accessories

NA

### Test Setup

Default

### Test Execution

This test verify the BT chip  detection of  Texas Instrument

Sample log for BT test is shown below

===================================

BLUETOOTH Test

===================================

Detecting Bluetooth chip

Detected a Texas Instruments BT Chip  with an ID = 0xd

Chip detectation completed

-------------------------x----------------------------

## USB Test

### Test Accessories

USB mass storage device

### Test Setup

Connect the USB mass storage device (for e.g pen drive ) to DM8127 CSK

### Test Execution

This test verifies USB interface access on DM8127 CSK. Below operations are executed as part of this test

- Reading USB Manufacturer Name,Vendor ID & Product ID.

- Reading USB device class ID and device type.

- Device acts as a USB Host.

Sample USB test log is shown below

===================================

USB Test

===================================

MUSB revision: 0x4EA20800

PHY ON

Waiting for connect

Connected

Manufacturer Name::  Storage Media

VENDOR ID :: 0x54c

PRODUCT ID :: 0x2a5

DEVICE class ID :: 0x8

Device Type :: MASS STORAGE

Checking Speed and cable

USBSS VBUS Stable

USBSS is a Host

Cable is Type A

USBSS is a High/Full Speed Device

-------------------------X----------------------------

## LED Test

### Test Accessories

NA

### Test Setup

Default

### Test Execution

This test blinks the five onboard GPIO LED’s in a cyclic fashion.

Sample log for LED test is shown below

===================================

LED Test

===================================

LED step and blinking test performing

-------------------------X----------------------------

## Switch Test

### Test Accessories

NA

### Test Setup

Default

### Test Execution

This test verifies the switch detection and the position of the switch is displayed on the console. This test is cyclic and can be tested for various positions of the switch.

Sample log for switch test is shown below

===================================

Switch Test

===================================

Press any push button switch on the DM8127 CSK and check

if the corresponding switch name is displayed on the console

SW2 is pressed

Press X/x for exiting the test

Press Y/y to continue

y

continue

SW3 is pressed

Press X/x for exiting the test

Press Y/y to continue

y

continue

SW4 is pressed

Press X/x for exiting the test

Press Y/y to continue

x

Exiting from the push button test

Push Button Test Completed!

-------------------------X----------------------------

## WLAN Test

### Test Accessories

NA

### Test Setup

Default

### Test Execution

This test verifies the WL1837 WLAN Chip Detected with Manufacturer ID of 0x40760097

Sample log for WLAN test is shown below

===================================

WLAN Test

===================================

Test communicates with an SDIO WLAN Chip and displays its ChipID

Detected an SDIO Card with 2 functions

SDIO Card Detected

WL1271 WLAN Chip Detected with an Manufacturer ID of 0x40760097

WLAN Test Passed

-------------------------X----------------------------

## Camera Test

### Test Accessories

Parallel camera module

### Test Setup

Connect the camera module to camera connector of DM8127 CSK

### Test Execution

This test configure the camera sensor for proper capturing of video data and shall be validated by observing the PCLK, HSYNC &VSYNC signals on oscilloscope.

Sample log for Camera test is shown below

===================================

CAMERA Test

===================================

lvds configuration completed

ar0331_configuration completed

-------------------------X----------------------------

## HDMI Test

### Test Accessories

HDMI Monitor and HDMI cable

### Test Setup

Connect the HDMI monitor to HDMI connector of DM8127 CSK

### Test Execution

This test display the video color bar pattern on HDMI monitor after making the target connection to CORTEX-A8

Note: Actual results are to be judged based on the manual observation

## Composite Video Test

### Test Accessories

HDMI Monitor and HDMI cable

### Test Setup

Connect the HDMI monitor to HDMI connector of DM8127 CSK

### Test Execution

This test display the video color bar pattern on HDMI monitor after making the target connection to CORTEX-A8

Note: Actual results are to be judged based on the manual observation.

# Appendix

## Creating Ethernet Loopback Cable

Take UTP LAN cable and cut it way from connector side. Wire any one of the side as per below diagram