PSpice LM5026 Instruction Manual

## To Extract the Files:

## Unzip LM5026 Customer Package.zip to any root directory. The following files are located, unless otherwise specified, in the LM5026 Customer Package folder.

- LM5026 Eval Board.opj: The PSpice project file used to open the evaluation board design in OrCAD.

- LM5026 EVAL BOARD.DSN: The PSpice V16.3 evaluation board schematics for simulating startup, steady state, line transient disturbances, or load transient disturbances.

- LM5026 EVAL BOARD V16PT2 .DSN: The evaluation board schematics formatted for PSpice V16.2.

- eval board.lib: PSpice .subckt and .model library definitions for evaluation board components.

- lm5026.lib: the PSpice .subckt library definition of the LM5026 model.

- EVAL BOARD.OLB: Schematic symbols for eval board.lib.

- LM5026.OLB: Schematic symbols for LM5026.lib.

- *.net, *.out: PSpice generates readable netlists and spice output listings in the file structure rooted at LM5026 Customer Package\LM5026 Eval Board-PspiceFiles. See, for example

##### LM5026 Customer Package\LM5026 Eval Board-PSpiceFiles\startup\startup.net

and  LM5026 Customer Package\LM5026 Eval Board-PSpiceFiles\startup\startup\startup.out

[note the nested startup folders].

## Installation Instructions:

## To simulate the evaluation board:

## Open the project file LM5026 Eval Board.opj with OrCAD Capture. The PSpice COM wrapper error message may appear but it can usually be ignored.

## If OrCAD fails to open the project click Help > About OrCAD Capture to check the version number. For v16.2, click File > Open > Design and select LM5026 EVAL BOARD V16PT2.DSN.

## To simulate a new design:

## Select or generate a project folder and copy lm5026.lib and LM5026.OLB to that folder by copying from the LM5026 Customer Package folder.

## Open OrCAD and click File > New > Project to create a new project based on empty_all_libs.opj.

## In the OrCAD Project Manager tab, add the LM5026 to the list of library files.

## Select library and right-click Add file.

## Navigate to the project folder to open LM5026.OLB.

## Create a root schematic and open the empty schematic page to begin placing and wiring parts.

## To specify the simulation and set error controls Click Pspice > New Simulation Profile.

## To duplicate simulation controls from the evaluation board project, use the browse button next to the Inherit From box to select an existing .sim file such as LM5026 Customer Package\LM5026 Eval Board-PSpiceFiles\startup\startup.sim

## Use the Analysis tab to select the Analysis Type and specify associated analysis directives.

## Use the Configuration Files tab to establish the path to lm5026.lib.

## Select the Library category and use the browse button next to the Filename box to navigate to the LM5026 Customer Package folder.

## Double click lm5026.lib.

## Click the Add to Design button to make this specification global to the entire project.

  - You can generate additional profiles as needed. The library is automatically configured. Use the Inherit From box to retain other settings.