LMK030xxC Evaluation Board

User's Guide

November 2013

SNAU040A

LMK03000C/01C/02C/33C

Precision Clock Conditioner with Integrated VCO

Evaluation Board Operating Instructions

April 2009

Table of Contents

Equipment	5

Basic Operation	6

LMK03000C Board information	9

Loop Filter #1	9

OSCin	10

Fout	10

Loop Filter	10

Features of the board	11

Other Important Notes	11

Evaluation Board Revision v1.0 Errata	11

Results	11

Phase Noise	11

Delays	13

VCO Performance	14

Impact of Reference on Phase Noise	16

LMK03001C Board information	17

Loop Filter #1	17

OSCin	18

Fout	18

Loop Filter	18

Features of the board	19

Other Important Notes	19

Evaluation Board Revision v1.0 Errata	19

Results	19

Phase Noise	19

Delays	21

VCO Performance	22

Impact of Reference on Phase Noise	24

LMK03002C Board information	25

Loop Filter #1	25

OSCin	26

Fout	26

Loop Filter	26

Features of the board	27

Other Important Notes	27

Results	27

Phase Noise	27

Delays	29

VCO Performance	30

Loop Filter #2	30

Impact of Reference on Phase Noise	32

LMK03033C Board information	33

Loop Filter #1	33

OSCin	33

Fout	34

Loop Filter	34

Features of the board	35

Other Important Notes	35

Evaluation Board Revision v1.0 Errata	35

Results	35

Phase Noise	35

Delays	37

VCO Performance	38

Appendix A: Schematics	40

Appendix B: Bill of Materials	43

Appendix C: Build Diagram	45

Appendix D: Quick Start on EVM Communication	47

## Equipment

Power Supply

The Power Supply should be a low noise power supply.  An Agilent 6623A Triple power supply with LC filters on the output to reduce noise was used in creating these evaluation board instructions.

Phase Noise / Spectrum Analyzer

For measuring phase noise an Agilent E5052A is recommended.  An Agilent E4445A PSA Spectrum Analyzer with the Phase Noise option is also usable although the architecture of the E5052A is superior for phase noise measurements.  At frequencies less than 100 MHz the local oscillator noise of the PSA is too high and measurements will be of the local oscillator, not the device under test.

Oscilloscope

The oscilloscope and probes should be capable of measuring the output frequencies of interest when evaluating this board. The Agilent Infiniium DSO81204A was used in creating these evaluation board instructions.

Reference Oscillator

The on board crystal oscillator will provide a low noise reference signal to the device at offsets greater than 1 kHz.

Note: The default loop filter has a loop bandwidth of ~60 kHz.  Inside the loop bandwidth of a PLL the noise is greatly affected by any noise on the reference oscillator (OSCin).  Therefore any noise on the oscillator less than 60 kHz will be passed through and seen on the outputs.  For this reason the main output of a Signal Generator is not recommended for driving OSCin in this setup.

## Basic Operation

- Connect a low noise 3.3 V power supply to the Vcc connector located at the top left of the board.

- Please see Appendix D for quick start on interfacing the board. Connect PC to the uWire header.

- Start CodeLoader4.exe.

- Select the USB or LPT Communication Mode on the Port Setup tab as appropriate.

- Enable output to be measured, any of CLKout(0-7) or EN_Fout from either Clock Outputs or Bits/Pins tab. In example below, CLKout0 is enabled.

| Program Bits |  |
| --- | --- |
| POWERDOWN | Powers the part down. |
| EN_Fout | Turns on the Fout pin for measuring the internal VCO. |
| OSCin_FREQ | Must be set to the OSCin frequency in MHz. |
| PLL_MUX | Programmable to many different values to support Lock Detect or aid troubleshooting. |
| DIV4 | Shall be checked for OSCin frequencies greater than 20 MHz. |
| RESET | The registers can be defaulted by checking and unchecking RESET. Software bits will not reflect this. |
| VCO_R3_LF VCO_R4_LF VCO_C3_C4_LF | Internal loop filter values, also accessible from Clock Outputs tab. |
| EN_CLKout0..7 | Enable CLKout bits from CLKout0 to CLKout7. Also accessible from Clock Outputs tab. |
| EN_CLKout_Global | Enable all clock outs. If unselected then the EN_CLKouts are overridden and the outputs are all disabled. |
| Program Pins |  |
| GOE | Set Global Output Enable to high or low logic level. |
| SYNC* | Set SYNC* pin to high or low logic level. |
| TRIGGER | Set auxiliary trigger pin to high or low logic level. |

- Program the part by clicking “Keyboard Controls”  “Load Device” OR by shortcut “Ctrl+L”.

- Make measurements…  After programming, the uWire cabling can be unplugged from the evaluation board to minimize noise and EMI.

## LMK03000C Board information

| ## Loop Filter #1 |  |  |  |
| --- | --- | --- | --- |
| Phase Margin | 70.0º | K | 3200 uA |
| Loop Bandwidth | 55.3 kHz | fPD | 9.72 MHz |
| Crystal Frequency | 19.44 MHz | Output Frequency | 1185 to 1296 MHz |
| Supply Voltage | 3.3 Volts | VCO Gain | 8 MHz/Volt |
| Loop filter #1 is selected by placing a 0 ohm resistor on pad R22. This loop filter has been designed for optimal RMS jitter using a low noise reference. |  |  |  |

### OSCin

By default the board is configured to use the on-board crystal oscillator.  It is also possible to use the board with a single ended or differential reference source at the OSCin port.  Below are several possible configurations for driving OSCin.

| OSCin using on board crystal oscillator [default] |  |
| --- | --- |
| 0 ohm | R8, R11, R20 [power to crystal oscillator], R109 |
| 39 ohm | R9 [can also be 0 ohm – depends on oscillator output power, 39 ohms to be a voltage divider] |
| 51 ohm | R15 |
| 0.1 uF | C35, C36 (C5 is a 0.1 uF 0402 cap which may be moved to C36) |
| Open | C4, C5 R7, R10, R12, R13, R14, R16, R17, R79, R112 |

| Differential OSCin setup |  |
| --- | --- |
| 0 ohm | R7, R8, R10, R13 |
| 100 ohm | R44 |
| 0.1 uF | C5, C35 (C36 is a 0.1 uF 0402 cap which may be moved to C5) |
| Open | C4, C36 R11, R12, R14, R15, R16, R79 R20 [remove power from crystal oscillator for noise reasons] |

| Single ended OSCin setup |  |
| --- | --- |
| 0 ohm | R7, R8 |
| 51 ohm | R15 |
| 0.1 uF | C35, C36 (C5 is a 0.1 uF 0402 cap which may be moved to C36) |
| Open | C4, C5 R10, R11, R12, R13, R14, R16, R17, R79 R20 [remove power from crystal oscillator for noise reasons] |

### Fout

Fout allows direct access to the internal VCO before the clock distribution section.  The EN_Fout bit must be selected to enable Fout.  A 3 dB pad is placed on R80, R81, and R82.

### Loop Filter

R22 and R5 form a “resistor switch” which allows either one of two different loop filters to be selected.

| Loop Filter | Resistor Switch | Loop Filter Components | Default Loop Bandwidth |
| --- | --- | --- | --- |
| Loop Filter #1 [default] | R22 Shorted | C1, C2, C2p, R2 | 59.1 kHz |
| Loop Filter #2 | R5 Shorted | C1_AUX, C2_AUX, C2p_AUX, R2_AUX | 77 Hz |

### Features of the board

- Either one of two loop filters can be selected by shorting either R22 or R5.  More info about each loop filter can be found in the General Description and Appendix A.

- Test points for each of the uWire lines are scattered in the lower left corner of the board and include: GOE_TP, DATAuWire, CLKuWire, LEuWire, SYNC_TP, and LD_TP.

- Ground is located on the unstuffed 10 pin header on the left side of the board.

- Ground is located on the GND_tp2 in the upper left corner of the board and GND_tp1 located to the right of the Vcc SMA connector.

- Ground is located on the bottom side of the board on each pad of the unstuffed 10 pin header GND_J2.

- Vcc is located on the unstuffed 10 pin header on the upper left side of the board.

- Vcc is located on VccPlane test point located to the right of the Vcc SMA.

- Vcc is located on the bottom side of the board on each pad of the unstuffed 10 pin header VCC_J2

### Other Important Notes

- When changing the OSCin frequency, the OSCin frequency register needs to be changed to match.

- Toggle the SYNC* pin to synchronize the clock outputs when in divided mode.

- For both loop filters, a helper silkscreen is offset from the loop filters to help identify the components according to Texas Instruments Incorporated’s traditional reference designators associated with loop filters.

### Evaluation Board Revision v1.0 Errata

- SYNC* is labeled on the PCB as SYNC, however the logic of SYNC* is still active low!

### Results

| ## Phase Noise |  |  |
| --- | --- | --- |
| Output Frequency = 1244.16 MHz Internal VCO, Fout output |  | Reference source is on board 19.44 MHz crystal Below ~1 kHz phase noise is dominated by the crystal 10 Hz – 20 MHz integrated RMS jitter = 2.1 ps 100 Hz – 20 MHz integrated RMS jitter = 287 fs 12 kHz – 20 MHz integrated RMS jitter = 236 fs |
| LVDS output CLKout0 VCO Frequency = 1244.16 MHz, VCO_DIV=2, CLKout0_div=4 LVDS output (155.52 MHz) |  | Output is measured with a Minicircuits ADT2-1T balun. Reference source is on board 19.44 MHz crystal Below ~1 kHz phase noise is dominated by the crystal 10 Hz – 20 MHz integrated RMS jitter = 1.8 ps 100 Hz – 20 MHz integrated RMS jitter = 315 fs 12 kHz – 20 MHz integrated RMS jitter = 256 fs (shown) |
| LVPECL output CLKout4 VCO Frequency = 1244.16 MHz, VCO_DIV=2, CLKout4_div=4 LVPECL output (155.52 MHz) |  | Output is measured with a Minicircuits ADT2-1T balun. Reference source is on board 19.44 MHz crystal Below ~1 kHz phase noise is dominated by the crystal 10 Hz – 20 MHz integrated RMS jitter = 1.7 ps 100 Hz – 20 MHz integrated RMS jitter = 281 fs 12 kHz – 20 MHz integrated RMS jitter = 244 fs (shown) |

| ## Delays |  |
| --- | --- |
| These delay measurements illustrate how skew errors due to different length traces may be tuned out. The delay may be adjusted in steps of 150 ps. |  |
|  | Delays 150, 300, 450, 600, 750 |
|  | CLKout0_DLY = 0 ps CLKout1_DLY = all delays programmed: 0, 150, 300, 450, 600, 750, 900, 1050, 1200, 1350, 1500, 1650, 1800, 1950, 2100, and 2250 ps |

## VCO Performance

The internal VCO performance is measured by using a narrow bandwidth loop filter.  By default the narrow loop bandwidth filter is stuffed as Loop Filter #2 in positions C1_AUX, C2_AUX, C2p_AUX, and R2_AUX and has a loop bandwidth of 77 Hz.

See the Loop Filter section in Board Options for more detail about switching between the two different loop filters.

| Loop Filter |  |  |  |
| --- | --- | --- | --- |
| Phase Margin | 58.7º | K | 100 uA |
| Loop Bandwidth | 77 Hz | fPD | 1.023 MHz |
| Crystal Frequency | 19.44 MHz | Output Frequency | 1185 to 1296 MHz |
| Supply Voltage | 3.3 Volts | VCO Gain | 10 MHz/Volt |
| This loop filter is located on the top side of the PCB and is selected by placing a 0 ohm resistor on pad R5. This loop filter has been designed with a very small loop bandwidth to minimize the PLL from interacting with the noise of the VCO to permit a VCO phase noise measurement. |  |  |  |

VCO Phase Noise - Narrow Loop Bandwidth

This plot shows the noise of the VCO at 1240 MHz using a 1 MHz Phase Detector Frequency. An external oscillator was used for this plot, since the VCO noise dominates, reference oscillator noise is not critical. The loop bandwidth has been minimized so that the VCO is the dominant noise contributor. 10 Hz – 20 MHz integrated RMS jitter = 56.9 ps (shown) 100 Hz – 20 MHz integrated RMS jitter = 25.0 ps 12 kHz – 20 MHz integrated RMS jitter = 0.304 ps (datasheet)

## Impact of Reference on Phase Noise

Inside the loop bandwidth of a PLL the phase noise is set by the quality of the reference oscillator used.  For this reason it is important to select a reference oscillator suitable for the application.

Test Setup

Using the same loop filter as described in the General Description and by driving the OSCin frequency with an ultra low jitter 100 MHz Wetzel Crystal (501-04517D) and setting R = 10 to achieve a phase detector frequency of 10 MHz.  A very low integrated RMS jitter of 247 fs is measured vs. the 2.1 ps measured in the Phase Noise section with 19.44 MHz crystal in the bandwidth of 10 Hz to 20 MHz.

10 Hz – 20 MHz integrated RMS jitter = 247 fs (shown)

100 Hz – 20 MHz integrated RMS jitter = 243 fs

12 kHz – 20 MHz integrated RMS jitter = 222 fs

Conclusion

This diagram illustrates how the phase noise inside the loop bandwidth is set by the quality of the reference oscillator used.  Phase noise outside the loop bandwidth is set by the VCO noise level.

## LMK03001C Board information

| ## Loop Filter #1 |  |  |  |
| --- | --- | --- | --- |
| Phase Margin | 74.4º | K | 3200 uA |
| Loop Bandwidth | 59.1 kHz | Fcomp | 12.288 MHz |
| Crystal Frequency | 12.288 MHz | Output Frequency | 1470 to 1570 MHz |
| Supply Voltage | 3.3 Volts | VCO Gain | 10 MHz/Volt |
| Loop filter #1 is selected by placing a 0 ohm resistor on pad R22. This loop filter has been designed for optimal RMS jitter using a low noise reference. |  |  |  |

### OSCin

By default the board is configured to use the on-board crystal oscillator.  It is also possible to use the board with a single ended or differential reference source at the OSCin port.  Below are several possible configurations for driving OSCin.

| OSCin using on board crystal oscillator [default] |  |
| --- | --- |
| 0 ohm | R8, R11, R20 [power to crystal oscillator], R109 |
| 39 ohm | R9 [can also be 0 ohm – depends on oscillator output power, 39 ohms to be a voltage divider] |
| 51 ohm | R15 |
| 0.1 uF | C35, C36 (C5 is a 0.1 uF 0402 cap which may be moved to C36) |
| Open | C4, C5 R7, R10, R12, R13, R14, R16, R17, R79, R112 |

| Differential OSCin setup |  |
| --- | --- |
| 0 ohm | R7, R8, R10, R13 |
| 100 ohm | R44 |
| 0.1 uF | C5, C35 (C36 is a 0.1 uF 0402 cap which may be moved to C5) |
| Open | C4, C36 R11, R12, R14, R15, R16, R79 R20 [remove power from crystal oscillator for noise reasons] |

| Single ended OSCin setup |  |
| --- | --- |
| 0 ohm | R7, R8 |
| 51 ohm | R15 |
| 0.1 uF | C35, C36 (C5 is a 0.1 uF 0402 cap which may be moved to C36) |
| Open | C4, C5 R10, R11, R12, R13, R14, R16, R17, R79 R20 [remove power from crystal oscillator for noise reasons] |

### Fout

Fout allows direct access to the internal VCO before the clock distribution section.  The EN_Fout bit must be selected to enable Fout.  A 3 dB pad is placed on R80, R81, and R82.

### Loop Filter

R22 and R5 form a “resistor switch” which allows either one of two different loop filters to be selected.

| Loop Filter | Resistor Switch | Loop Filter Components | Default Loop Bandwidth |
| --- | --- | --- | --- |
| Loop Filter #1 [default] | R22 Shorted | C1, C2, C2p, R2 | 59.1 kHz |
| Loop Filter #2 | R5 Shorted | C1_AUX, C2_AUX, C2p_AUX, R2_AUX | 77 Hz |

### Features of the board

- Either one of two loop filters can be selected by shorting either R22 or R5.  More info about each loop filter can be found in the General Description and Appendix A.

- Test points for each of the uWire lines are scattered in the lower left corner of the board and include: GOE_TP, DATAuWire, CLKuWire, LEuWire, SYNC_TP, and LD_TP.

- Ground is located on the unstuffed 10 pin header on the left side of the board.

- Ground is located on the GND_tp2 in the upper left corner of the board and GND_tp1 located to the right of the Vcc SMA connector.

- Ground is located on the bottom side of the board on each pad of the unstuffed 10 pin header GND_J2.

- Vcc is located on the unstuffed 10 pin header on the upper left side of the board.

- Vcc is located on VccPlane test point located to the right of the Vcc SMA.

- Vcc is located on the bottom side of the board on each pad of the unstuffed 10 pin header VCC_J2

### Other Important Notes

- When changing the OSCin frequency, the OSCin frequency register needs to be changed to match.

- Toggle the SYNC* pin to synchronize the clock outputs when in divided mode.

- For both loop filters, a helper silkscreen is offset from the loop filters to help identify the components according to Texas Instruments Incorporated’s traditional reference designators associated with loop filters.

### Evaluation Board Revision v1.0 Errata

- SYNC* is labeled on the PCB as SYNC, however the logic of SYNC* is still active low!

### Results

| ## Phase Noise |  |  |
| --- | --- | --- |
| Output Frequency = 1499.136 MHz Internal VCO, Fout output |  | Reference source is on board 12.288 MHz crystal Below ~1 kHz phase noise is dominated by the crystal 10 Hz – 20 MHz integrated RMS jitter = 782 fs 100 Hz – 20 MHz integrated RMS jitter = 269 fs 12 kHz – 20 MHz integrated RMS jitter = 238 fs |
| LVDS output CLKout0 VCO Frequency = 1499.136 MHz, INPUT_DIV=2, CLKout0_div=4 LVDS output (187.392 MHz) |  | Output is measured with a Minicircuits ADT2-1T balun. Reference source is on board 12.288 MHz crystal Below ~1 kHz phase noise is dominated by the crystal 10 Hz – 20 MHz integrated RMS jitter = 702 fs 100 Hz – 20 MHz integrated RMS jitter = 277 fs 12 kHz – 20 MHz integrated RMS jitter = 245 fs (shown) |
| LVPECL output CLKout4 VCO Frequency = 1499.136 MHz, INPUT_DIV=2, CLKout4_div=4 LVPECL output (187.392 MHz) |  | Output is measured with a Minicircuits ADT2-1T balun. Reference source is on board 12.288 MHz crystal Below ~1 kHz phase noise is dominated by the crystal 10 Hz – 20 MHz integrated RMS jitter = 679 fs 100 Hz – 20 MHz integrated RMS jitter = 278 fs 12 kHz – 20 MHz integrated RMS jitter = 248 fs (shown) |

| ## Delays |  |
| --- | --- |
| These delay measurements illustrate how skew errors due to different length traces may be tuned out. The delay may be adjusted in steps of 150 ps. |  |
|  | Delays 150, 300, 450, 600, 750 |
|  | CLKout0_DLY = 0 ps CLKout1_DLY = all delays programmed: 0, 150, 300, 450, 600, 750, 900, 1050, 1200, 1350, 1500, 1650, 1800, 1950, 2100, and 2250 ps |

## VCO Performance

The internal VCO performance is measured by using a narrow bandwidth loop filter.  By default the narrow loop bandwidth filter is stuffed as Loop Filter #2 in positions C1_AUX, C2_AUX, C2p_AUX, and R2_AUX and has a loop bandwidth of 77 Hz.

See the Loop Filter section in Board Options for more detail about switching between the two different loop filters.

| Loop Filter |  |  |  |
| --- | --- | --- | --- |
| Phase Margin | 58.7º | K | 100 uA |
| Loop Bandwidth | 77 Hz | Fcomp | 1.024 MHz |
| Crystal Frequency | 12.288 MHz | Output Frequency | 1470 to 1570 MHz |
| Supply Voltage | 3.3 Volts | VCO Gain | 10 MHz/Volt |
| This loop filter is located on the top side of the PCB and is selected by placing a 0 ohm resistor on pad R5. This loop filter has been designed with a very small loop bandwidth to minimize the PLL from interacting with the noise of the VCO to permit a VCO phase noise measurement. |  |  |  |

VCO Phase Noise - Narrow Loop Bandwidth

This plot shows the noise of the VCO at 1500 MHz using a 1 MHz Phase Detector Frequency. An external oscillator was used for this plot, since the VCO noise dominates, reference oscillator noise is not critical. The loop bandwidth has been minimized so that the VCO is the dominant noise contributor. 10 Hz – 20 MHz integrated RMS jitter = 53.1 ps (shown) 100 Hz – 20 MHz integrated RMS jitter = 26.3 ps 12 kHz – 20 MHz integrated RMS jitter = 0.310 ps

## Impact of Reference on Phase Noise

Inside the loop bandwidth of a PLL the phase noise is set by the quality of the reference oscillator used.  For this reason it is important to select a reference oscillator suitable for the application.

Test Setup

Using the same loop filter as described in the General Description and by driving the OSCin frequency with an ultra low jitter 100 MHz Wetzel Crystal (501-04517D) and setting R = 10 to achieve a phase detector frequency of 10 MHz.  A very low integrated RMS jitter of 243 fs is measured vs. the 782 fs measured in the Phase Noise section with 12.288 MHz crystal in the bandwidth of 10 Hz to 20 MHz.

10 Hz – 20 MHz integrated RMS jitter = 243 fs (shown)

100 Hz – 20 MHz integrated RMS jitter = 240 fs

12 kHz – 20 MHz integrated RMS jitter = 220 fs

Conclusion

This diagram illustrates how the phase noise inside the loop bandwidth is set by the quality of the reference oscillator used.  Phase noise outside the loop bandwidth is set by the VCO noise level.

## LMK03002C Board information

| ## Loop Filter #1 |  |  |  |
| --- | --- | --- | --- |
| Phase Margin | 63º | K | 3200 uA |
| Loop Bandwidth | 105 kHz | Fcomp | 16 MHz |
| Crystal Frequency | 16 MHz | Output Frequency | 1566 to 1724 MHz |
| Supply Voltage | 3.3 Volts | VCO Gain | 13 MHz/Volt |
| (Loop filter #1 is selected by placing a 0 ohm resistor on pad R22) |  |  |  |

### OSCin

By default the board is configured to use the on-board crystal oscillator.  It is also possible to use the board with a single ended or differential reference source at the OSCin port.  Below are several possible configurations for driving OSCin.

| OSCin using on board crystal oscillator [default] |  |
| --- | --- |
| 0 ohm | R8, R11, R20 [power to crystal oscillator], R109 |
| 39 ohm | R9 [can also be 0 ohm – depends on oscillator output power, 39 ohms to be a voltage divider] |
| 51 ohm | R15 |
| 0.1 uF | C35, C36 (C5 is a 0.1 uF 0402 cap which may be moved to C36) |
| Open | C4, C5 R7, R10, R12, R13, R14, R16, R17, R79, R112 |

| Differential OSCin setup |  |
| --- | --- |
| 0 ohm | R7, R8, R10, R13 |
| 100 ohm | R17 |
| 0.1 uF | C5, C35 (C36 is a 0.1 uF 0402 cap which may be moved to C5) |
| Open | C4, C36 R11, R12, R14, R15, R16, R79 R20 [remove power from crystal oscillator for noise reasons] |

| Single ended OSCin setup |  |
| --- | --- |
| 0 ohm | R7, R8 |
| 51 ohm | R15 |
| 0.1 uF | C35, C36 (C5 is a 0.1 uF 0402 cap which may be moved to C36) |
| Open | C4, C5 R10, R11, R12, R13, R14, R16, R17, R79 R20 [remove power from crystal oscillator for noise reasons] |

### Fout

Fout allows direct access to the internal VCO before the clock distribution section.  The EN_Fout bit must be selected to enable Fout.  A 3 dB pad is placed on R80, R81, and R82.

### Loop Filter

R22 and R5 form a “resistor switch” which allows either one of two different loop filters to be selected.

| Loop Filter | Resistor Switch | Loop Filter Components | Default Loop Bandwidth |
| --- | --- | --- | --- |
| Loop Filter #1 [default] | R22 Shorted | C1, C2, C2p, R2 | 105 kHz |
| Loop Filter #2 | R5 Shorted | C1_AUX, C2_AUX, C2p_AUX, R2_AUX | 55 Hz |

### Features of the board

- Either one of two loop filters can be selected by shorting either R22 or R5.  More info about each loop filter can be found in the General Description and Appendix A.

- Test points for each of the uWire lines are scattered in the lower left corner of the board and include: GOE_TP, DATAuWire, CLKuWire, LEuWire, SYNC_TP, and LD_TP.

- Ground is located on the unstuffed 10 pin header on the left side of the board.

- Ground is located on the GND_tp2 in the upper left corner of the board and GND_tp1 located to the right of the Vcc SMA connector.

- Ground is located on the bottom side of the board on each pad of the unstuffed 10 pin header GND_J2.

- Vcc is located on the unstuffed 10 pin header on the upper left side of the board.

- Vcc is located on VccPlane test point located to the right of the Vcc SMA.

- Vcc is located on the bottom side of the board on each pad of the unstuffed 10 pin header VCC_J2

### Other Important Notes

- When changing the OSCin frequency, the OSCin frequency register needs to be changed to match.

- Toggle the SYNC* pin to synchronize the clock outputs when in divided mode.

- For both loop filters, a helper silkscreen is offset from the loop filters to help identify the components according to Texas Instruments Incorporated’s  traditional reference designators associated with loop filters.

### Results

| ## Phase Noise |  |  |
| --- | --- | --- |
| Output Frequency = 1499.136 MHz Internal VCO, Fout output |  | Reference source is on board 12.288 MHz crystal Below ~1 kHz phase noise is dominated by the crystal 10 Hz – 20 MHz integrated RMS jitter = 782 fs 100 Hz – 20 MHz integrated RMS jitter = 269 fs 12 kHz – 20 MHz integrated RMS jitter = 238 fs |
| LVDS output CLKout0 VCO Frequency = 1499.136 MHz, INPUT_DIV=2, CLKout0_div=4 LVDS output (187.392 MHz) |  | Output is measured with a Minicircuits ADT2-1T balun. Reference source is on board 12.288 MHz crystal Below ~1 kHz phase noise is dominated by the crystal 10 Hz – 20 MHz integrated RMS jitter = 702 fs 100 Hz – 20 MHz integrated RMS jitter = 277 fs 12 kHz – 20 MHz integrated RMS jitter = 245 fs (shown) |
| LVPECL output CLKout4 VCO Frequency = 1499.136 MHz, INPUT_DIV=2, CLKout4_div=4 LVPECL output (187.392 MHz) |  | Output is measured with a Minicircuits ADT2-1T balun. Reference source is on board 12.288 MHz crystal Below ~1 kHz phase noise is dominated by the crystal 10 Hz – 20 MHz integrated RMS jitter = 679 fs 100 Hz – 20 MHz integrated RMS jitter = 278 fs 12 kHz – 20 MHz integrated RMS jitter = 248 fs (shown) |

| ## Delays |  |
| --- | --- |
| These delay measurements illustrate how skew errors due to different length traces may be tuned out. The delay may be adjusted in steps of 150 ps. |  |
|  | Delays 150, 300, 450, 600, 750 |
|  | CLKout0_DLY = 0 ps CLKout1_DLY = all delays programmed: 0, 150, 300, 450, 600, 750, 900, 1050, 1200, 1350, 1500, 1650, 1800, 1950, 2100, and 2250 ps |

## VCO Performance

The internal VCO performance is measured by using a narrow bandwidth loop filter.  By default the narrow loop bandwidth filter is stuffed as Loop Filter #2 in positions C1_AUX, C2_AUX, C2p_AUX, and R2_AUX and has a loop bandwidth of 55 Hz.

See the Loop Filter section in Board Options for more detail about switching between the two different loop filters.

| ### Loop Filter #2 |  |  |  |
| --- | --- | --- | --- |
| Phase Margin | 58º | K | 100 uA |
| Loop Bandwidth | 55 Hz | Fcomp | 500 kHz |
| Crystal Frequency | 16 MHz | Output Frequency | 1566 to 1724 MHz |
| Supply Voltage | 3.3 Volts | VCO Gain | 13 MHz/Volt |
| This loop filter is located on the top side of the PCB and is selected by placing a 0 ohm resistor on pad R5. This loop filter has been designed with a very small loop bandwidth to minimize the PLL from interacting with the noise of the VCO to permit a VCO phase noise measurement. |  |  |  |

VCO Phase Noise – Narrow Loop Bandwidth

This plot shows the noise of the VCO at 1632 MHz using a 500 kHz Phase Detector Frequency. An external oscillator was used for this plot, since the VCO noise dominates, reference oscillator noise is not critical. The loop bandwidth has been minimized so that the VCO is the dominant noise contributor. 10 Hz – 20 MHz integrated RMS jitter = 107.6 ps 100 Hz – 20 MHz integrated RMS jitter = 28.1 ps (shown) 1 kHz – 20 MHz integrated RMS jitter = 2.7 ps 12 kHz – 20 MHz integrated RMS jitter = 0.303 ps

## Impact of Reference on Phase Noise

Inside the loop bandwidth of a PLL the phase noise is set by the quality of the reference oscillator used.  For this reason it is important to select a reference oscillator suitable for the application.

Test Setup

Using the same loop filter as described in the General Description and by driving the OSCin frequency with an ultra low jitter 100 MHz Wetzel Crystal (501-04517D) and setting R = 5 to achieve a phase detector frequency of 20 MHz.  A very low integrated RMS jitter of 201 fs is measured vs. the 474 fs measured in the Phase Noise section with 16 MHz crystal in the bandwidth of 10 Hz to 20 MHz.

10 Hz – 20 MHz integrated RMS jitter = 201 fs (shown)

100 Hz – 20 MHz integrated RMS jitter = 197 fs

1 kHz – 20 MHz integrated RMS jitter = 196 fs

12 kHz – 20 MHz integrated RMS jitter = 188 fs

Conclusion

This diagram illustrates how the phase noise inside the loop bandwidth is set by the quality of the reference oscillator used.  Phase noise outside the loop bandwidth is set by the VCO noise level.

## LMK03033C Board information

| ## Loop Filter #1 |  |  |  |
| --- | --- | --- | --- |
| Phase Margin | 63.5 deg | K | 3200 uA |
| Loop Bandwidth | 104 kHz | Fcomp | 12.288 MHz |
| Crystal Frequency | 12.288 MHz | Output Frequency | 1843 to 2160 MHz |
| Supply Voltage | 3.3 Volts | VCO Gain | 20 MHz/Volt |

### OSCin

By default the board is configured to use the on-board crystal oscillator.  It is also possible to use the board with a single ended or differential reference source at the OSCin port.  Below are several possible configurations for driving OSCin.

| OSCin using on board crystal oscillator [default] |  |
| --- | --- |
| 0 ohm | R8, R11, R20 [power to crystal oscillator], R109 |
| 39 ohm | R9 [can also be 0 ohm – depends on oscillator output power, 39 ohms to be a voltage divider] |
| 51 ohm | R15 |
| 0.1 uF | C35, C36 (C5 is a 0.1 uF 0402 cap which may be moved to C36) |
| Open | C4, C5 R7, R10, R12, R13, R14, R16, R17, R79, R112 |

| Differential OSCin setup |  |
| --- | --- |
| 0 ohm | R7, R8, R10, R13 |
| 100 ohm | R44 |
| 0.1 uF | C5, C35 (C36 is a 0.1 uF 0402 cap which may be moved to C5) |
| Open | C4, C36 R11, R12, R14, R15, R16, R79 R20 [remove power from crystal oscillator for noise reasons] |

| Single ended OSCin setup |  |
| --- | --- |
| 0 ohm | R7, R8 |
| 51 ohm | R15 |
| 0.1 uF | C35, C36 (C5 is a 0.1 uF 0402 cap which may be moved to C36) |
| Open | C4, C5 R10, R11, R12, R13, R14, R16, R17, R79 R20 [remove power from crystal oscillator for noise reasons] |

### Fout

Fout allows direct access to the internal VCO before the clock distribution section.  The EN_Fout bit must be selected to enable Fout.  A 3 dB pad is placed on R80, R81, and R82.

### Loop Filter

R22 and R5 form a “resistor switch” which allows either one of two different loop filters to be selected.

| Loop Filter | Resistor Switch | Loop Filter Components | Default Loop Bandwidth |
| --- | --- | --- | --- |
| Loop Filter #1 [default] | R22 Shorted | C1, C2, C2p, R2 |  |
| Loop Filter #2 | R5 Shorted | C1_AUX, C2_AUX, C2p_AUX, R2_AUX |  |

### Features of the board

- Either one of two loop filters can be selected by shorting either R22 or R5.  More info about each loop filter can be found in the General Description and Appendix A.

- Test points for each of the uWire lines are scattered in the lower left corner of the board and include: GOE_TP, DATAuWire, CLKuWire, LEuWire, SYNC_TP, and LD_TP.

- Ground is located on the unstuffed 10 pin header on the left side of the board.

- Ground is located on the GND_tp2 in the upper left corner of the board and GND_tp1 located to the right of the VCC SMA connector.

- Ground is located on the bottom side of the board on each pad of the unstuffed 10 pin header GND_J2.

- VCC is located on the unstuffed 10 pin header on the upper left side of the board.

- VCC is located on VCCPlane test point located to the right of the VCC SMA.

- VCC is located on the bottom side of the board on each pad of the unstuffed 10 pin header VCC_J2

### Other Important Notes

- When changing the OSCin frequency, the OSCin frequency register needs to be changed to match.

- Toggle the SYNC* pin to synchronize the clock outputs when in divided mode.

- For both loop filters, a helper silkscreen is offset from the loop filters to help identify the components according to Texas Instruments Incorporated’s traditional reference designators associated with loop filters.

### Evaluation Board Revision v1.0 Errata

- SYNC* is labeled on the PCB as SYNC, however the logic of SYNC* is still active low!

### Results

| ## Phase Noise |  |  |
| --- | --- | --- |
| Output Frequency = 1966.08 MHz Internal VCO, Fout output |  | Reference source is on board 12.288 MHz crystal Below ~100 Hz phase noise is dominated by the crystal 100 Hz – 20 MHz integrated RMS jitter = 256 fs 12 kHz – 20 MHz integrated RMS jitter = 239 fs (shown) |
| LVDS output CLKout0 VCO Frequency = 1966.08 MHz, VCO_DIV=2, CLKout0_div=4 LVDS output (245.76 MHz) |  | Output is measured with a Minicircuits ADT2-1T balun. Reference source is on board 12.288 MHz crystal Below ~100 Hz phase noise is dominated by the crystal 100 Hz – 20 MHz integrated RMS jitter = 261 fs 12 kHz – 20 MHz integrated RMS jitter = 244 fs (shown) |
| LVPECL output CLKout4 VCO Frequency = 1966.08 MHz, VCO_DIV=2, CLKout4_div=4 LVPECL output (245.76 MHz) |  | Output is measured with a Minicircuits ADT2-1T balun. Reference source is on board 12.288 MHz crystal Below ~100 Hz phase noise is dominated by the crystal 100 Hz – 20 MHz integrated RMS jitter = 265 fs 12 kHz – 20 MHz integrated RMS jitter = 248 fs (shown) |

| ## Delays |  |
| --- | --- |
| These delay measurements illustrate how skew errors due to different length traces may be tuned out. The delay may be adjusted in steps of 150 ps. |  |
|  | Delays 150, 300, 450, 600, 750 |
|  | CLKout0_DLY = 0 ps CLKout1_DLY = all delays programmed: 0, 150, 300, 450, 600, 750, 900, 1050, 1200, 1350, 1500, 1650, 1800, 1950, 2100, and 2250 ps |

## VCO Performance

The internal VCO performance is measured by using a narrow bandwidth loop filter.  By default the narrow loop bandwidth filter is stuffed as Loop Filter #2 in positions C1_AUX, C2_AUX, C2p_AUX, and R2_AUX and has a narrow loop bandwidth.

See the Loop Filter section in Board Options for more detail about switching between the two different loop filters.

| Loop Filter |  |  |  |
| --- | --- | --- | --- |
| Phase Margin |  | K | 100 uA |
| Loop Bandwidth | <100 Hz | Fcomp | 1.2288 MHz |
| Crystal Frequency | 12.288 MHz | Output Frequency | 1843 to 2160 MHz |
| Supply Voltage | 3.3 Volts | VCO Gain | 20 MHz/Volt |
| This loop filter is located on the top side of the PCB and is selected by placing a 0 ohm resistor on pad R5. This loop filter has been designed with a very small loop bandwidth to minimize the PLL from interacting with the noise of the VCO to permit a VCO phase noise measurement. |  |  |  |

VCO Phase Noise - Narrow Loop Bandwidth

This plot shows the noise of the VCO at 1966.08 MHz using a 614.4 MHz Phase Detector Frequency. An external oscillator was used for this plot, since the VCO noise dominates, reference oscillator noise is not critical. The loop bandwidth has been minimized so that the VCO is the dominant noise contributor. 100 Hz – 20 MHz integrated RMS jitter = 58.8 ps (shown)

## Appendix A: Schematics

## Appendix B: Bill of Materials

| Part | Manufacturer | Part Number | Qnt | Identifier |
| --- | --- | --- | --- | --- |
| Capacitors |  |  |  |  |
| 47 pF | Kemet | C0603C470J5GAC | 1 | C68 |
| 100 pF | Kemet | C0402C101J5GAC | 14 | C20, C21, C22, C23, C24, C25, C26, C27, C28, C29, C30, C31, C32, C33 |
| 100 pF | Kemet | C0603C101J5GAC | 1 | C34 |
| 12 nF | Kemet | C0603C123K1RACTU | 1 | C2 |
| 0.1 uF | Kemet | C0603C104J3RAC | 16 | C9, C37, C38, C39, C40, C41, C42, C43, C44, C45, C46, C47, C48, C49, C50, C51 |
| 0.1 uF | Kemet | C0402C104J4RAC | 18 | C35, C36, C52, C53, C54, C55, C56, C57, C58, C59, C60, C61, C62, C63, C64, C65, C66, C67 |
| 820 nF | Kemet | C0603C824K8PAC | 1 | C1_AUX |
| 1 uF | Kemet | C0603C105K8VAC | 16 | C69, C70, C71, C72, C73, C74, C75, C76, C77, C79, C80, C81, C82, C83, C84, C85 |
| 10 uF | Kemet | C0805C106K9PAC | 5 | C2p_AUX, C16, C17, C18, C19 |
| 10 uF | Kemet | C0805C106K9PAC | 1 | C78 |
| Resistors |  |  |  |  |
| 0 ohm | Vishay | CRCW0603000ZRT1 | 10 | R1, R3, R6, R11, R12, R20, R22, R95, R104, R109 |
| 0 ohm | Yageo | RC0805JR-070RL | 2 | R4, R8 |
| 0 ohm | Vishay | CRCW0603000ZRT1 | 2 | R87, R94 |
| 18 ohm | Vishay | CRCW0603180JRT1 | 1 | R82 |
| 39 ohm | Vishay | CRCW0603390JRT1 | 1 | R9 |
| 51 ohm | Vishay/Dale | CRCW060351R0JNEA | 1 | R15 |
| 120 ohm | Vishay | CRCW0402120RJNED | 10 | R52, R53, R96, R97, R98, R99, R100, R101, R102, R103 |
| 180 ohm | Vishay | CRCW0603181JRT1 | 2 | R92, R93 |
| 270 ohm | Vishay | CRCW0603271JRT1 | 2 | R80, R81 |
| 820 ohm | Vishay | CRCW0603821JRT1 | 1 | R2_AUX |
| 1.8 k | Vishay/Dale | CRCW06031K80JNEA | 1 | R2 |
| 2.2 k | Vishay/Dale | CRCW06032K20JNEA | 2 | R29, R30 |
| 15 k | Vishay | CRCW0603153JRT1 | 4 | R88, R89, R90, R91 |
| 27 k | Vishay | CRCW0603273JRT1 | 3 | R83, R84, R85 |
| Other |  |  |  |  |
| LMK030xxC | Texas Instruments | LMK03000C/LMK03001C/ LMK03002C/LMK03033C | 1 | U1 |
| OSC - CRYSTEK 33xx | Crystek | C3391-19.440 | 1 | Y1 |
| ADT2-1T | Minicircuits | ADT2-1T | 1 | B3 |
| SMA | Johnson Components | 142-0701-851 | 11 | CLKout0, CLKout0*, CLKout2*, CLKout4, CLKout4*, CLKout7, CLKout7*, Fout, OSCin, OSCin*, Vcc |
| 3.3 V zener | Comchip | CZRU52C3V3 | 2 | D1, D2 |
| PCB_LMK0300x | Printed Circuits Corp | PCB_LMK0300x rev 1.1, 6-16-2007 | 1 | F1 |
| HEADER_2X5(POLARIZED) | FCI Electronics | 52601-S10-8 | 1 | uWire |
| SPCS-8 | SPC Technology | SPCS-8 | 4 | Standoffs in the four corners (insert from bottom) |
| Open |  |  |  |  |
| Open | - | Open | 2 | B1, B2 |
| Open | - | 603 | 32 | C1, C2_AUX, C6, C7, C8, C10, C11, C12, C13, C14, C15, R5, R7, R10, R14, R16, R17, R18, R19, R21, R23, R24, R25, R26, R27, R28, R79, R105, R107, R110, R111, R112 |
| Open | - | 805 | 1 | C2p |
| Open | - | Open | 1 | C3 |
| Open | - | 402 | 16 | C4, C5, R31, R32, R37, R38, R39, R44, R45, R46, R51, R58, R63, R68, R73, R78 |
| Open | - | 603 | 34 | C86, R33, R34, R35, R36, R40, R41, R42, R43, R47, R48, R49, R50, R54, R55, R56, R57, R59, R60, R61, R62, R64, R65, R66, R67, R69, R70, R71, R72, R74, R75, R76, R77, R86 |
| OpenSMA | - | Open | 13 | CLKout1, CLKout1*, CLKout2, CLKout3, CLKout3*, CLKout5, CLKout5*, CLKout6, CLKout6*, Fout*, GOE, LD, SYNC* |
| Open | - | HEADER_2X5 | 2 | J2, J4 |
| Open | - | HEADER_2X5 | 2 | J3, J5 |
| Open | - | 805 | 3 | R13, R106, R108 |
| Open | - | Open | 1 | Y2 |

## Appendix C: Build Diagram

# Appendix D: Quick Start on EVM Communication

Codeloader is the software used to communicate with the EVM (Please download the latest version from TI.com - http://www.ti.com/tool/codeloader). This EVM can be controlled through the uWire interface on board. There are two options in communicating with the uWire interface from the computer.

OPTION 1

Open Codeloader.exe  Click “Select Device”  Click “Port Setup” tab  Click “LPT” (in Communication Mode)

OPTION 2

The Adapter Board

This table describes the pins configuration on the adapter board for each EVM board (See examples below table)

| EVM | Jumper Bank |  |  |  |  |  |  |  | Code Loader Configuration |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | A | B | C | D | E | F | G | H |  |
| LMX2581 | A4 | B1 | C2 |  | E5 | F1 | G1 | H1 | BUFEN (pin 1), Trigger (pin 7) |
| LMX2541 | A4 |  | C3 |  | E4 | F1 | G1 | H1 | CE (pin 1), Trigger (pin 10) |
| LMK0400x | A0 |  | C3 |  | E5 | F1 | G1 | H1 | GOE (pin 7) |
| LMK01000 | A0 |  | C1 |  | E5 | F1 | G1 | H1 | GOE (pin 7) |
| LMK030xx | A0 |  | C1 |  | E5 | F1 | G1 | H1 | SYNC (pin 7) |
| LMK02000 | A0 |  | C1 |  | E5 | F1 | G1 | H1 | SYNC (pin 7) |
| LMK0480x | A0 | B2 | C3 |  | E5 | F0 | G0 | H1 | Status_CLKin1 (pin 3) |
| LMK04816/4906 | A0 | B2 | C3 |  | E5 | F0 | G0 | H1 | Status_CLKin1 (pin 3) |
| LMK01801 | A0 | B4 | C5 |  | E2 | F0 | G0 | H1 | Test (pin 3), SYNC0 (pin 10) |
| LMK0482x (prelease) | A0 | B5 | C3 | D2 | E4 | F0 | G0 | H1 | CLKin1_SEL (pin 6), Reset (pin 10) |
| LMX2531 | A0 |  |  |  | E5 | F2 | G1 | H2 | Trigger (pin 1) |
| LMX2485/7 | A0 |  | C1 |  | E5 | F2 | G1 | H0 | ENOSC (pin 7), CE (pin 10) |
| LMK03200 | A0 |  |  |  | E5 | F0 | G0 | H1 | SYNC (pin 7) |
| LMK03806 | A0 |  | C1 |  | E5 | F0 | G0 | H1 |  |
| LMK04100 | A0 |  | C1 |  | E5 | F1 | G1 | H1 |  |

Example adapter configuration (LMK01801)

Open Codeloader.exe  Click “Select Device”  Click “Port Setup” Tab  Click “USB” (in Communication Mode)

*Remember to also make modifications in “Pin Configuration” Section according to Table above.