# LP8863-Q1

Functional Safety Failure In Time (FIT) Rates

Table 1. Component Failure Rates per IEC TR 62380 / ISO 26262 Part 11

| FIT IEC TR 62380 / ISO 26262 | FIT (Failures Per 109 Hours) |
| --- | --- |
| Total FIT Rate | 25 |
| Die FIT Rate | 5 |
| Package FIT Rate | 20 |

The failure rate and mission profile information in Table 1 comes from the Reliability data handbook IEC

TR 62380 / ISO 26262 part 11:

• Mission Profile: Motor Control from Table 11

• Power dissipation: 600 mW

• Climate type: World-wide Table 8

• Package factor lambda 3 Table 17b

• Substrate Material: FR4

• EOS FIT rate assumed: 0 FIT

Table 2. Component Failure Rates per Siemens Norm SN 29500-2

| Table | Category | Reference FIT Rate | Reference Virtual TJ |
| --- | --- | --- | --- |
| 5 | CMOS/BICMOS ASICs Analog & Mixed =<50V supply | 70 FIT | 70 C |

The Reference FIT Rate and Reference Virtual TJ (junction temperature) in Table 2 come from the

Siemens Norm SN 29500-2 tables 1 through 5. Failure rates under operating conditions are calculated

from the reference failure rate and virtual junction temperature using conversion information in SN 29500-

2 section 4.