DM388 SOM CSK

Diagnostics User Manual

| CHANGE HISTORY |  |  |  |  |
| --- | --- | --- | --- | --- |
| Version No. | Date | Change Description | Author | Remarks |
| 0.1 | 12-Oct-16 | Created | Akhil |  |
|  | 17-Oct-16 | Modified Test log format ,Test description, Added HDMI Interface test | Akhil |  |
|  | 17-Nov-16 | Changed diagnostic Test Procedure section to Diagnostic Flow and updated Test section | Akhil |  |

| TEMPLATE CHANGE HISTORY |  |  |  |
| --- | --- | --- | --- |
| Version No. | Date | Change Description | Remarks |
| 1.00 | 20-Jul-11 | Initial Baseline |  |

Table of Contents

1 INTRODUCTION	4

1.1 Purpose and Scope	4

1.2 Acronyms	4

2 Overview	5

2.1 Diagnostics Flow	6

2.2 Default Test Setup	12

2.3 Test Accessories	12

2.4 DDR3L Test	13

2.4.1 Test Accessories	13

2.4.2 Test Setup	13

2.4.3 Test Execution	13

2.5 Nand Flash Test	14

2.5.1 Test Accessories	14

2.5.2 Test Setup	14

2.5.3 Test Execution	14

2.6 USB Test	15

2.6.1 Test Accessories	15

2.6.2 Test Setup	15

2.6.3 Test Execution	15

2.7 LED Test	16

2.7.1 Test Accessories	16

2.7.2 Test Setup	16

2.7.3 Test Execution	16

2.8 SD Card Test	16

2.8.1 Test Accessories	16

2.8.2 Test Setup	16

2.8.3 Test Execution	16

2.9 WLAN Test	17

2.9.1 Test Accessories	17

2.9.2 Test Setup	17

2.9.3 Test Execution	17

2.10 BT Test	17

2.10.1 Test Accessories	17

2.10.2 Test Setup	17

2.10.3 Test Execution	17

2.11 Audio Test	18

2.11.1 Test Accessories	18

2.11.2 Test Setup	18

2.11.3 Test Execution	18

2.12 Switch Test	19

2.12.1 Test Accessories	19

2.12.2 Test Setup	19

2.12.3 Test Execution	19

2.13 Ethernet Test	20

2.13.1 Test Accessories	20

2.13.2 Test Setup	20

2.13.3 Test Execution	20

2.14 Camera Test	21

2.14.1 Test Accessories	21

2.14.2 Test Setup	21

2.14.3 Test Execution	21

2.15 HDMI Test	21

2.15.1 Test Accessories	21

2.15.2 Test Setup	21

2.15.3 Test Execution	21

2.16 Composite Test	21

2.16.1 Test Accessories	21

2.16.2 Test Setup	21

2.16.3 Test Execution	21

3 Appendix	23

3.1 Creating Ethernet Loopback Cable	23

# INTRODUCTION

## Purpose and Scope

This document provides the details of DM388 diagnostic test software suite. The scope of this document is limited to the diagnostic software, its setup, compilation and test procedures on the DM388. It is assumed that the audience of this document is familiar with using the DM388 CSK Board and has access to the DM388 reference  document(TRM).

## Acronyms

| Acronyms | Description |
| --- | --- |
| DDR | Double Data Rate |
| LED | Light Emitting Diode |
| SD | Secure Digital |
| SoC | System on Chip |
| TRM | Technical Reference Manual |
| UART | Universal Asynchronous Receiver/Transmitter |
| USB | Universal Serial Bus |
| CCS | Code Composer Studio |
| IO | Input/Output |
| GPIO | General purpose input/output |
| WLAN | Wireless local area network |
| BT | Bluetooth |

# Overview

DM388 CSK diagnostics software provides a set of interface tests to validate the DM388 CSK hardware. Diagnostic tests enable quick verification of the interface connectivity and basic functionality before starting the development on DM388. Diagnostic software also serves as reference code for low level hardware configurations of the DM388 CSK and SoC. Diagnostic test suite is designed to execute from CCS on DM388 CSK along with Carrier card. Diagnostic tests use UART serial interface of SoC to output the test results and accept the user inputs.

Below picture shows the major interfaces on the DM388 CSK Board.

LINE-OUT

JACK (J9)

LINE-IN

JACK (J10)

ON/OFF SWITCH (SW1)

CLIENT USB CONN (J12)

5V DC

JACK (J1)

HDMI Connector (J6)

DEBUG USB CONN (J11)

CSI Camera (Expansion) Connector (J15)

Parallel Camera Module Connector (J7)

ETHERNET Connector (J8)

TMDSCSK388 Module

Reverse Voltage Indication LED (LD3)

POWER LED

(LD1)

FAULT INDICATION LED (LD2)

TVOUT

CONN (J13)

Boot mode Switch (SW1)

## Diagnostics Flow

- Connect 5v adapter to DM388 CSK

- Connect micro USB cable between the micro USB port of carrier card and host PC

- Connect all the Test accessories required for Interface testing(for e.g ethernet loopback cable,audio headset,HDMI cable e.t.c)

- Power “ON” the board

- Open CCS

- Successful execution of above step will open CCS as shown in above figure.

- Import diagnostic project

- Click  on project menu at the top and select “Import CCS Projects”

- Browse to the path to import diagnostic project

- Click finish

- Successful execution of above step will  create a project  on left side of CCS as shown in above figure.

- Build dm388 diagnostic project

- Execute the build command from CCS to build diagnostic project.

- Successful execution of build will create diagnostic test binaries(i.e dm388csk_diagnostic.out) under the Path /dm388csk_diagnostic/Debug/dm388csk_diagnostic.out as shown in above figure.

- Open the DM388 target configuration file(dm388csk.ccxml)

- Click  on View menu at the top and select “Target Configurations”

- Successful Execution of above step will show the DM388 target configuration file(dm388csk.ccxml) on the right hand side of CCS as shown in above figure.

- Right click on DM388 target configuration file(dm388csk.ccxml) and select “Launch 	Selected Configuration”

- Successful Execution of above step will show the launched DM388 target configuration

- Make target connection to Cortex- A8 Processor

- Right click on Cortex- A8 Processor and select “connect Target”

- Reset the CPU

- Click  on Run menu at the top and go to reset

- Click on “CPU SW Reset”

- Open serial console (Ex: Teraterm) on host PC, connect to COM port on which BoosterPack Debug port is connected and setup for following configurations

    - Baud rate -    115200

    - Data length -  8 bit

    - Parity -          None

    - Stop bits -      1

    - Flow control -  None

  - Press the ENTER Key on Keyboard to get the console message as shown below.

- Load and run “dm388csk_diagnostic.out

*******************************************************************

DM388 CSK HW Diagnostic Tests

*******************************************************************

Diagnostic Tests       			 Pass		 Fail

-----------------------			------		------

0 - Auto Run All Tests            	  0		  0

1 - NAND Flash Test               	  0		  0

2 - DDR Test                      	  0		  0

3 - USB Test                      	  0		  0

4 - LED Test                      	  0		  0

5 - SD Card Test                  	  0		  0

6 - WLAN Test                     	  0		  0

7 - Bluetooth Test                	  0		  0

8 - Audio Test                    	  0		  0

9 - Switch Test                   	  0		  0

10 - Ethernet Test                 	  0		  0

11 - Camera Test                   	  0		  0

q - Quit

Enter Desired Option:

- Execute a particular test by entering the test number. Procedure to run each test is described in following sections.

## Default Test Setup

- Connect DM388 CSK board to carrier card

- Connect 5V power supply to the board

- Connect micro USB cable  between debug port of carrier card and host PC

## Test Accessories

Below are the test accessories required for executing the diagnostic tests

- DM388 CSK board

- DM388 CSK carrier card

- Power supply(5v & 3A)

- Windows Host PC

- Micro USB cable

- Ethernet Loopback cable

- HDMI monitor

- HDMI cable

- Audio LINE cable & Headphone

- SD Card

- Micro USB to type A adaptor

- USB device (USB stick, mouse etc)

- Camera module

## DDR3L Test

### Test Accessories

NA

### Test Setup

Default

### Test Execution

This test verifies DDR3L interface access on DM388 CSK. Below operations are executed as part of this test

- Read and Write “0xAAAAAAAA” pattern test for DDR[0].

- Read and Write  “0x55555555” pattern test for DDR[0].

- Read and Write  “0xAAAAAAAA” pattern test for DDR[1].

- Read and Write  “0x55555555” pattern test for DDR[1].

- Read and Write Incremental pattern test for DDR[0].

- Read and Write  Incremental pattern test for DDR[1].

Sample DDR3 test log is shown below

********************************

DDR3 Test

********************************

Carrying out 0xAAAAAAAA pattern test for DDR[0].

Carrying out 0x55555555 pattern test for DDR[0].

Carrying out 0xAAAAAAAA pattern test for DDR[1].

Carrying out 0x55555555 pattern test for DDR[1].

Carrying out Incremental pattern test for DDR[0].

Carrying out Incremental pattern test for DDR[1].

DDR3 Test Passed!

DDR3 Tests Completed!!

## Nand Flash Test

### Test Accessories

NA

### Test Setup

Default

### Test Execution

This test verifies NAND flash interface access on DM388 CSK. Below operations are executed as part of this test

- Reading NAND flash device information

- Erasing NAND flash blocks

- Writing data to NAND flash page

- Reading data from NAND flash page and comparing the data with written data for correctness

- Verifications of NAND flash write protect

Sample NAND Flash test log is shown below

********************************

Nandflash Test

********************************

Running NAND Flash Chip Detect Test

NAND MFGID = 2c

NAND DEVID = ca

NAND Flash Chip Detect Test Passed

Running NAND Flash Block Erase Test

Testing 4 blocks

Erasing NAND blocks

NAND Erase successful.

NAND Erase successful.

NAND Erase successful.

NAND Erase successful.

--> 0 block erase errors

NAND Flash Block Erase Test Passed

Running NAND Flash Memory Access Test

Programming NAND pages

Comparing data

--> 0 page verify errors

NAND Flash Memory Access Test Passed

Running NAND Flash Write Protect Test

Testing NAND write protection

Checking to see if data has been protected

NAND Flash Write Protect Test Passed

Nand Flash Passed!

Nand Flash Tests Completed!!

-----------x----------

## USB Test

### Test Accessories

USB mass storage device

### Test Setup

Connect the USB mass storage device (for e.g pen drive ) to DM388 CSK

### Test Execution

This test verifies USB interface access on DM388 CSK. Below operations are executed as part of this test

- Reading USB Manufacturer Name,Vendor ID & Product ID.

- Reading USB device class ID and device type.

- Device acts as a USB Host.

Sample USB test log is shown below

*****************************

USB Test

*****************************

Running USB Detect Test

MUSB revision: 0x4EA20800

phy_on..

Waiting for connect

Connected

Manufacturer Name:: POLLEX 4G

VENDOR ID :: 0x90c

PRODUCT ID :: 0x1000

DEVICE class ID :: 0x8

Device Type :: MASS STORAGE

USBSS VBUS Stable

USBSS is a Host

USB Detect Test Completed!

USB Test Completed!!

-----------x----------

## LED Test

### Test Accessories

NA

### Test Setup

Default

### Test Execution

This test  blinks the five onboard GPIO LED’s in a cyclic fashion.

Sample log for LED test is shown below

************************

Led Test

************************

Running LED Toggle Test

LED Toggle Test Completed

However, actual results are to be judged based on the observation

LED Test Completed!!

## SD Card Test

### Test Accessories

SD Card

### Test Setup

Insert  SD Card  to the  DM388 CSK

### Test Execution

This test verifies SD card interface access on DM388 CSK. Below operations are executed as part of this test

- SD card detection

- Writing data to a block/sector

- Reading data from a block/sector

- Verification of read/write data

Sample log for SD test is shown below

***********************

SD Test

***********************

SD Card Detected

Running SD Card Read/Write Test

SDMMC: Writing a pattern to Block 2....Write test passed

SDMMC: Reading from Block 2...Read test passed

Verify data...Passed

SD Card Read/Write Test Passed

SD Card Test Passed!

SD Card Test Completed!!

-----------x----------

## WLAN Test

### Test Accessories

NA

### Test Setup

Default

### Test Execution

This test verify the  WL1837 WLAN Chip Detected with an Manufacturer ID of 0x40760097

Sample log for WLAN test is shown below

*************************

WLAN Test

*************************

WLAN Chip Enabled

Running WLAN Chip Detect Test

Detected an SDIO Card with 2 functions

SDIO Card Detected

WL1837 WLAN Chip Detected with an Manfacturer ID of 0x40760097

WLAN Chip Detect Test Passed

WLAN Test Passed!

WLAN Test Completed!!

-----------x----------

## BT Test

### Test Accessories

NA

### Test Setup

Default

### Test Execution

This test verify the  BT chip  detection of  Texas Instrument

Sample log for BT test is shown below

********************************

Bluetooth Test

********************************

Detected a Texas Instruments BT Chip

BT Test Passed

Bluetooth Test Completed!!

-----------x---------

## Audio Test

### Test Accessories

LINE IN cable & headset

### Test Setup

- Connect the LINE IN cable to audio out of  test  PC and  LINE IN port of DM388 CSK

- Connect the headset to LINE OUT of DM388 CSK.

### Test Execution

This test verify that Audio supplied from test PC  through  LINE IN cable at LINE IN port  is Coming to Headphone which is connected to LINE OUT of  DM388 CSK.

Sample log for Audio test is shown below

******************************

AIC3104 Audio Codec Test

******************************

Running Audio Line in Test

Verify that Audio supplied at LINE IN port is Coming to Headphone

Audio Line in Test Completed

AIC3104 Audio Codec Test Completed!!

-----------x----------

## Switch Test

### Test Accessories

NA

### Test Setup

Default

### Test Execution

This test verifies the switch detection and the position of the switch is displayed on the console. This test is cyclic and can be tested for various positions of the switch.

Sample log for  switch test is shown below

***************************

Switch Test

***************************

Running Switch Position Test

Press any  switch on the dm388 board and check

if the corresponding switch name is displayed on the console

SW5 is pressed

Press X/x for exiting the test

Press Y/y to continue

y

continue

SW8 is pressed

Press X/x for exiting the test

Press Y/y to continue

x

Exiting from the  Switch Position test

Switch Position Test Completed

Switch Test Completed!!

-----------x----------

## Ethernet Test

### Test Accessories

Ethernet loopback cable

### Test Setup

Connect the Ethernet loopback cable to the RJ-45 connector

### Test Execution

This test verifies Ethernet interface access on DM388 CSK. Below operations are executed as part of this test

- Sending and recieving packet via loopback cable

- Verifying phy internal loopback operation

- Verifying phy external loopback operation

Sample log for Ethernet loopback test is shown below

*****************************

Ethernet Test

*****************************

PHY found at address 25

Testing PHY internal loopback

Packet verification passed

PHY internal loopback test completed

Testing Ethernet external loopback

Waiting for link...

Link Detected

Packet verification passed

Ethernet external loopback test passed

Ethernet Test Passed!

Ethernet Test Completed!!

-----------x----------

## Camera Test

### Test Accessories

Parallel camera module

### Test Setup

Connect the camera module to camera connector of  DM388 CSK

### Test Execution

This test configure the camera sensor for proper capturing of  video data and shall be validated by observing  the  PCLK,HSYNC &VSYNC signals on oscilloscope.

Sample log for Camera test is shown below

********************************

Camera Test

********************************

lvds configuration completed

ar0331_configuration completed

Camera Test Completed!!

-----------x----------

Note: However, actual results are to be judged based on the observation

## HDMI Test

### Test Accessories

HDMI Monitor and HDMI cable

### Test Setup

Connect the HDMI monitor to HDMI connector of  DM388 CSK

### Test Execution

This test display the video color bar pattern on HDMI monitor after Making the target connection to CORTEX-A8

Note: However, actual results are to be judged based on the observation

## Composite Test

### Test Accessories

HDMI Monitor and composite video cable

### Test Setup

Connect the HDMI monitor to composite video connector of  DM388 CSK

### Test Execution

This test display the video color bar pattern on HDMI monitor after Making the target connection to CORTEX-A8

Note: However, actual results are to be judged based on the observation

# Appendix

## Creating Ethernet Loopback Cable

Take UTP LAN cable and cut it way from connector side. Wire any one of the side as per below diagram