| CUSTOMER | TI |
| --- | --- |
| CARD NAME | TIDA-00315-E1 |
| PART NUMBER | TIDA-00315-E1 |
| DATE | 05/21/15 |
| DESIGNED AT | Tessolve Semiconductor Pvt. Ltd. |

| GERBER FORMAT | RS274X , ENGLISH 2,5 LEADING |
| --- | --- |
| DRILL FORMAT | EXCELLON ENGLISH 2,5 TRAILING |
| PACKAGE NAME | TS11418AC0_TIDA-00315-E1_FAB_20150521.zip |

LIST OF FILES INSIDE PACKAGE:

ELECTRICAL LAYERS :

TIDA-00315.GTL	- Layer 1 (Top Side)

TIDA-00315.GBL	- Layer 2 (Bottom Side)

NON ELECTRICAL LAYERS :

TIDA-00315.GTS	- Solder Mask Top Side

TIDA-00315.GBS	- Solder Mask Bottom Side

TIDA-00315.GTO	- Silkscreen Top Side

TIDA-00315.GBO	- Silkscreen Bottom Side

TIDA-00315.GM1	- Board Outline

TIDA-00315.GD1	- Fabrication Drawing

DRILL LAYERS :

TIDA-00315.TXT		- Plated and Non Plated through holes drill file

OTHER FILES :

Fabrication Testpoint Report for TIDA-00315.ipc 	- IPC-D-356 Netlist

Layers.pdf		- PDF Plots of all layers

Fab.pdf		- PDF of Fabrication Drawing

Readme_Fab.doc	- This file

BOARD SPECIFICATIONS :

Board Size				 : 150 mm X 66.80 mm

No. of Layers			 :	2

Board Thickness			 : 63 mil  +/- 10%

Board Finish			 : ENIG

Min. Trace width and Spacing  : 15 mil / 8.4 mil

Min. Hole Size			 : 7.874 mil

Impedance Control		 : NA